|  | CSLR61 : EMBEDDED SYSTEMS LAB-2 |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- |
|  |  |  | ## Roll no. : 106119100 ## Name : Rajneesh Pandey ## Section : CSE-B |  |  |  |  |

- Switch between hex counter and decade counter using switches. Display the current value of the counter with the help of 4 LEDs and the screen.

•Libraries Used: InterruptIn, BusOut

#include "mbed.h"

BusOut leds(LED1, LED2, LED3, LED4);

InterruptIn swt(p5);

void hexCounter()

{

while (1)

{

for (int i = 0; i < 16; i++)

{

leds = i;

printf("Hex count : %d\n", i);

wait(1);

}

}

}

void decCounter()

{

while (1)

{

for (int i = 0; i < 10; i++)

{

leds = i;

printf("Dec count : %d\n", i);

wait(1);

}

}

}

int main()

{

swt.rise(&hexCounter);

swt.fall(&decCounter);

}

Output :

- Display the octal counter using the onboard LEDs using Ticker Object.

After 10 second, reset all the values of the onboard LED using Timeout Object.

•Libraries Used: TimeOut, Ticker

#include "mbed.h"

BusOut leds(LED1, LED2, LED3, LED4);

Ticker tck;

Timeout to;

int octCount = 0;

void octalCounter()

{

leds = octCount;

printf("counter  %d\n", leds.read());

octCount = (octCount + 1) % 8;

}

void reset()

{

printf("resetting...");

octCount = 0;

leds = 0;

printf("Value after resetting  %d\n", leds.read());

}

int main()

{

tck.attach(&octalCounter, 2);

to.attach(&reset, 10);

}

- Display the odd counter using the onboard LEDs and screen; After 10 seconds, reset all the values of the onboard LED and display even counter using Timeout Object. Give switch option to manually toggle between two counters.

Libraries Used: InterruptIn, Ticker, TimeOut, BusOut

#include "mbed.h"

BusOut leds(LED1, LED2, LED3, LED4);

Ticker tck;

Timeout to;

InterruptIn swt(p5);

int state = 1;

int odd = 1;

void Counter(){

while(1){

for(int i = 0; i < 16; i++){

if (i % 2 == state){

leds = i;

if (state)

printf("Odd Counter %d\n", leds.read());

else{

printf("Even Counter %d\n", leds.read());

}

wait(1);

}

}

}

}

void Toggle(){

state = !state;

printf("Toggle using switch...\n");

Counter();

}

void reset(){

printf("resetting Odd Counter...\n");

state = 0;

leds = 0;

tck.detach();

printf("Starting Even Counter...\n");

Counter();

}

void oddCounter(){

leds = odd;

odd = (odd+2)%16;

printf("Odd Counter %d\n", leds.read());

}

int main()

{

tck.attach(&oddCounter, 2.0);

to.attach(&reset, 10);

swt.rise(&Toggle);

}