| Date : 18/04/2022 |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- |
|  | CSLR61 : EMBEDDED SYSTEMS LAB-8 |  |  |  |  |  |  |
|  |  |  | ## Roll no.: 106119100 ## Name: Rajneesh Pandey ## Section: CSE-B |  |  |  |  |

1. Interface force sensor with Arduino board and display the amount of force given as input to sensor in LCD screen. Also, interface LED RGB and change colour of the LED based on the some threshold values.

Link

https://www.tinkercad.com/things/5DcUnGz3VKH-106119100-lab8-q1/editel?sharecode=JHq59qpZhKhb5HB661qiAM9QHw7z_XDrp2QK9USTIS4

// 106119100 Rajneesh Pandey

#include <LiquidCrystal.h>

LiquidCrystal lcd(12, 11, 10, 9, 8, 7);

int force = 0;

int R = 4;

int B = 3;

int G = 2;

void setup()

{

lcd.begin(16, 2);

pinMode(A0, INPUT);

pinMode(G, OUTPUT);

pinMode(R, OUTPUT);

pinMode(B, OUTPUT);

Serial.begin(9600);

}

void loop()

{

force = analogRead(A0);

Serial.println(force);

lcd.clear();

lcd.print(force);

if (force < 100)

{

digitalWrite(G, HIGH);

digitalWrite(B, LOW);

digitalWrite(R, LOW);

}

else if (force < 300)

{

digitalWrite(G, LOW);

digitalWrite(B, HIGH);

digitalWrite(R, LOW);

}

else

{

digitalWrite(R, HIGH);

digitalWrite(G, LOW);

digitalWrite(B, LOW);

}

delay(1000);

}

2. Interface the keypad and tilt sensor with Arduino board, if the sensor is being tilted, then take input from keypad and print it in the LCD.

Link

https://www.tinkercad.com/things/8yT2Vd8ZaEb-106119100-lab8-q2/editel?sharecode=cTdYGK3DKBbGd5nclB7T03VDdGdG2jeujSZRl4OOThE

// 106119100 Rajneesh Pandey

#include <LiquidCrystal.h>

#include <Keypad.h>

const byte numRows = 4; // number of rows on the keypad

const byte numCols = 4; // number of columns on the keypad

// keymap defines the key pressed according to the row and columns just as appears on the keypad

char keymap[numRows][numCols] =

{

{'1', '2', '3', 'A'},

{'4', '5', '6', 'B'},

{'7', '8', '9', 'C'},

{'*', '0', '#', 'D'}};

// Code that shows the the keypad connections to the arduino terminals

byte rowPins[numRows] = {10, 9, 8, 7};    // Rows 0 to 3

byte colPins[numCols] = {A0, A1, A2, A3}; // Columns 0 to 3

// initializes an instance of the Keypad class

Keypad myKeypad = Keypad(makeKeymap(keymap), rowPins, colPins, numRows, numCols);

LiquidCrystal lcd(12, 11, 5, 4, 3, 2);

int tilt = A4;

char buf[16] = {0};

int cnt = 0;

void setup()

{

Serial.begin(9600);

lcd.begin(16, 2);

pinMode(tilt, INPUT);

}

void loop()

{

int reading = analogRead(tilt);

char keypressed = myKeypad.getKey();

Serial.println(reading);

if (reading > 100)

{

buf[cnt] = keypressed;

if (keypressed != 0)

cnt++;

cnt %= 16;

buf[cnt] = 0;

lcd.clear();

lcd.print(buf);

}

else

{

lcd.clear();

lcd.print("IDLE");

}

delay(50);

}